[Location] Energy Consultation

Prepared For:

[Owner/Building Manager/whoever you talked to]

Prepared By:

[Author]

[Date]

## Introduction

The purpose of this energy consultation was to discover potential areas of energy reduction and savings within the [Place of consultation]. This consultation was conducted using a questionnaire and check list during a walk-through, with supplementary data on past gas and electric usage. [Talk about the place of audit and what service they provide/produce/ and who they serve. Talk about their building and what they have done to improve its energy efficiency]

[Talk about who did the report, their qualification, who they work for, and how they did the audit.]

Notice: While there has been an effort made to ensure that the information contained in this report is accurate, it should be taken into consideration that some of the information may be incomplete, inaccurate, or become out of date. Therefore, Mole Valley District Council, Worcester Polytechnic Institute, and all associated persons do not provide any guarantees on the information provided in the following report.

[Things that are in red (like this), should be either taken out or filled in. It should make sense what should be done. When the final report is handed in, there should be no red things in brackets.]

## Action Plan

The recommendations listed below are prioritized by payback period and estimated costs. Further explanations of each recommendation are provided.

| Priority | Recommendations | Estimated Annual Savings |  |  | Estimated Costs (£) | Payback Periods (years) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | (£) | CO2 (Kg) | (kWh) |  |  |
| 1 | Lighting Control | 375 | 3,656 | 6,699 | --- | --- |
| 2 | Additions to Building Management System | --- | --- | --- | --- | --- |
| 3 | LED Lighting | 520 | 2,940 | 5,390 | 2,494 | 4.8 |
| 4 | Envelope and Door Insulation | 3 | 16 | 30 | --- | --- |
| Total |  | 898 | 6,612 | 12,119 | 2,494 | 4.8 |

[This is a sample table, when your calculations are done, you can fill in your own numbers. This is placed here so that you can tell where the table needs to be placed.]

## Energy Savings

[More detailed tables should be included below. Every time something is calculated for savings, it should be added to another separate section. Below there are two sections for lighting, one is LED lighting and the other is lighting control. These calculations are a little different therefore; they are separated into different ECMs. Building envelope is usually tied together as one because increasing building envelope stops heat being lost. Sample tables are shown below along with sample site-specific descriptions from various case studies. Usually these are ranked according to the businesses needs, if a business has low gross income, and then the low cost solutions are suggested first, if a company has high gross income then the payback period is the top consideration.]

## Behavioral Changes

Behavior changes may seem small at the time of implementation, but they can add up to make a large difference in the long run. Make sure that all radiators are unobstructed and kept on appropriate settings. Obstructed radiators are forced to work harder to heat a room resulting in higher energy usage. Also a radiator that is left on its maximum setting will never turn off and over heat a room. Keeping radiators unobstructed and at a setting of 3 to 4 you will save energy and money. Turning off unnecessary lighting or choosing to work in the dark are more type of behavioral changes that can occur.

Site Specific Examples:

- In the front entrance the radiator is blocked by chairs and a table. Consider rearranging this area to allow the heating system to work more efficiently.

Figure 1: Chairs and table blocking radiator

## Lighting Control

Lights that are on when they are not needed consume extra energy. There are different situations which can result in lights being on, though they are not needed. Timed lights, lights in not continuously used areas, or incorrectly zoned lights are some examples of why more lights may be in use than are needed.

Site Specific Examples:

- Toilettes are frequently used areas, where the lights are left on when no one is in the room. All the toilettes in the building should have motion sensors installed on the lights. This will save energy by turning off the lights when the room is not being used, while adding the convenience of having the lights turn on when someone enters the area. For added employee convenience with motion sensors, check to see if it has a timer.

| Location | Total Number of Bulbs in Zoning Area | Watt of Bulb | Total Current kWh per Year | New Consumption with Technology | Money Saved per Year (£) | CO2 Savings (kg of CO2) | Cost of Appropriate Sensor (£) | Payback Period (Years) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Outside | 14 | 26 | 786 | 393 | 38 | 214 | 50 | 1.3 |
| Toilette | 14 | 50 | 1,512 | 378 | 37 | 618 | 50 | 1.4 |
| Toilette | 74 | 26 | 4,156 | 1,039 | 100 | 1699 | 50 | 0.5 |

- The lights in the open office areas of the building are zoned in square sections. These lights should be zoned such that the line of lights closest to the windows are one zone with subsequent zones moving further from the windows. If they are zoned in this manner, the lights closest to the windows can be turned off when the sun provides sufficient lighting, therefore saving energy. This means, with sufficient sunlight, approximately one eight to one quarter of a large room’s energy consumption can be cut, and one half of a small office can be cut.

| Total Number of Bulbs in Zoning Area | Total Current kWh per Year | New kWh if 1/4-1/8 of Lights are Turned Off | Money Saved per Year (£) | CO2 Savings (kg of CO2) |
| --- | --- | --- | --- | --- |
| 298 | 21,890 | 19,150-20,520 | 270-130 | 1,500-750 |
| *This is if the lights are off for half a day on average throughout the year |  |  |  |  |

## Energy Efficient Lighting

Replace existing lights in the facility with newer, more efficient lights. Many old lighting elements have become inefficient over the years. Replacing the existing elements with LED equivalents allows you to save significant amounts of money on your energy bill. If LED bulbs are not a viable option at the current time then CFL and fluorescent tubes can be used but make sure that the most efficient ones are in use i.e. T8 or T5’s for fluorescent tubes and bulbs with high energy ratings of A or B should be used. The table below breaks down the recommended areas where lighting should be replaced with all the correlating information about payback periods and costs.

Site Specific Examples:

- All halogen spotlights and CFL bulbs can be replaced with lower wattage LED lighting where possible.

| Old Light Bulb Wattage (W) | New Bulb Type | New Light Bulb Wattage (W)* | Price per New Bulb (£)* | Number of Bulbs Replacing | Price of Electricity (£)** | Total Bulb Costs | Savings per Year (£) | kWh Saved per Year (kWh) | Payback Period (years) | CO2 Savings per Year (kg CO2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50 | LED | 7 | 43 | 58 | 0.09669 | 2,494 | 520 | 5,390 | 4.80 | 2,940 |
| *All calculations are based off pricing and wattage from Halers Lighting 2011 Catalogue |  |  |  |  |  |  |  |  |  |  |
| **This is based off your current rates |  |  |  |  |  |  |  |  |  |  |

## Building Envelope

Inefficient building envelope or fabric can result in a loss of heating energy. The goal of upgrading building fabric is to separate outside air from infiltrating the building envelope. Large amounts of windows, holes or gaps in the building’s exterior and poor quality windows can all contribute to compromising the effectiveness of the overall building insulation.

Site Specific Examples:

- The single-pane windows and backdoor in the kitchen can be enhanced for better insulation. There are three options present here: Use thin film insulation over the windows, upgrade to double pane windows or completely remove windows and put up a well insulated concrete wall.

- Graphs to support choices

- The chimneys to the fireplaces are a contributor to cold air infiltrating the building envelope. Consider attaining a chimney flue to stop cold air from entering the building. Also, look into a more elaborate scheme to block the fireplace entrance when it is not being used. Cardboard is a fairly good insulator however the current insulating scheme does not seem to fit the fireplace hole very well.

- Exterior doors and the door to the W.C. should all be checked to ensure that rubber seals are in place and are stopping cold air penetration to the bar/lounge area.

- The old ventilation fans should be removed/filled. These holes are approximately one square meter and are located near the ceiling, so any hot air collected is leaked out through these old holes.

- Graphs for this

## Additions to Building Management System

Adding controls or sensors to a BMS will allow it to work less and, therefore save energy. Controlling the air flow and temperature in a building, especially a larger one, can be stressful and time-consuming for the facilities staff. The BMS is an automatic control that handles most of the managing load with timers and electronic thermostat settings. When the BMS has more information, it can work in a more efficient manner and help cut costs. Some devices can be added to the BMS to help it work in a smarter manner.

Site Specific Examples:

- Oxygen sensors measure the amount of oxygen extracted from a room, with these linked to a BMS, it could make way for a closed loop ventilation system. When the system senses a low-level of oxygen in the air, the outside air vent will open allowing for fresh air to mix in. When the vent is closed, old air is reused to make the system work less hard.

## Boiler Valve Insulation

The valves on heated pipes and the pipes themselves can lose heat and cause the heating system to work harder than necessary. Insulating these valves and pipes will increase the efficiency of the heating system and decrease carbon emissions.

Site Specific Examples:

- Consider insulating the valves on all heated pipes in the boiler room. You can do this on your own, buying the insulation, or have a company come in and give you a free quote to have the insulation done professionally. You may also be able to apply for Enhanced Capital Allowance (ECA), which will help pay for some of the insulation through tax breaks. For more information on ECA go to: www.eca.gov.uk/ and see Appendix A

## Thin Client Computers

The office’s desk top computers should be switched to newer, more efficient IGEL Thin Clients. IGEL Thin Clients use at least 50% less energy than a standard desktop computer. Replacing them in appropriate situations can result in high savings.

Site Specific Examples:

- The office already has and uses the Thin Client server, installing the Thin Client unit would cause minimal disruption.

- Computers in reception and offices do not use any technical programs that would require more than the IGEL offers. There are seven computers of this type.

- Twenty-four hour service computers will not be changed. There are six computers of this type.

- IGEL Thin Client computers consume much less energy than the currently used desktop computers.

## Downsize/Condense Energy Using Products

Using any number of smaller refrigerators instead of an equivalent sized unit consumes excess amounts of power. Having more refrigerators also occupies extra floor space, due to their geometry. Replacing multiple refrigerators with one larger unit would take up less floor space because the one unit will be taller instead of three wide small units.

Site Specific Examples:

- The refrigerator, freezer, and combination unit can be reduced to a single unit. To replace the current configuration, with a similar one, would cost around £200 for each new unit. These replacements would total 350 liters in storage capacity and have an average annual consumption of about 550 kWh per year. When replacing the three current units, look for a single combination unit with similar or larger capacity and less annual consumption.

## Additional Considerations

These are items that should be considered to help increase energy savings. [Often when calculations would be too inaccurate due to a plethora of assumptions or if they seem too implausible, they are mentioned as side notes so that the client still has the option to do them and can research about it a little bit more. The idea here is to plant the ECM in their mind and let them hopefully make something of it by either performing the ECM, researching for an alternative, or a way to think about the energy waste in a different manner]

Site Specific Examples:

- From brief conversation with Planning employees in the Pippbrook building, it seems as though the doors and windows located in the back of the building are not considered historic because they are located in the back of the building where they are not seen by the public. The staff offers free advice upon email containing a short description of the project including pictures of its location relative to the storefront.

- Fireplace heating is a very aesthetic and cozy option for heating, however much of the heat produced by the actual flame is leaked through the chimney. Another option would be to completely block up the chimney to stop cold air from infiltrating the building, however no fire will be present, and the pub may lose the cozy ambiance. The best option for fireplace heating is a wood pellet stove. This will keep the aesthetic of a fireplace however, will lose the sounds and actual feel of a raw fire. Not only will the sounds of a raw fire will be lost however, you will also lose the cold draft present from the chimney, which would help increase the building envelope the same way the previous option would.

- Some LED holiday lights are being used out in the garden. There are some that are not LEDs, consider replacing these with LED lights.

- Flush controllers for urinals in the Gentlemen toilets to sense motion or times used. A piece of technology you may wish to purchase that takes advantage of the ECA can be found here: http://goo.gl/yXELm. There is also another product from the same company with a few small changes that also applies, this can be found here: http://goo.gl/k9ADE.

- [There are others from case studies, these are just a few examples of what we mean by additional considerations…]

## Brief Summary

In summary, it can be seen that while this building is already doing very well in energy efficiency, there are still areas which can be improved to help reduce energy usage. Many of these steps are of a higher cost or more difficulty, but will be worth the change due to the amount saved on energy bills. Some savings opportunities in this report have not been quantified due to the large amount of variables present in the calculation. Carefully read over the report and take advantage of the most useful suggestions first. We recommend that you seek the opinion of another expert to check our numbers and suggestions. Good luck and happy savings!

Appendix A

Enhanced Capital Allowance (ECA)

ECA allows for companies who are investing in energy saving technologies to receive a tax relief on their profits for the year based on the amount they spent on the energy saving technologies. This provide companies to have additional capital to be able to put energy saving equipment in that would normally be too expensive so that both the environment and the company are able to benefit quicker from their investments.

There are currently many energy saving technologies out there for the many different sectors of a business from lighting, to HVAC, to pipe insulation, to motors, etc. The technologies for these sectors can often provide great energy savings but can cost more than a business is willing to spend. The ECA then provides a company with additional capital so that they can benefit from the savings of the energy efficient technology.

About ECA: This link provides more information on why the ECA was started and some of the key features that it has.

http://www.eca.gov.uk/etl/about/

How ECA works: This link breaks down how exactly the ECA works and how much money a person may see from claiming an ECA.

http://www.eca.gov.uk/etl/about/How+does+the+ECA+scheme+work.htm

Benefits of ECA: This link provides more information how claiming an ECA can benefit a business from cash-flow boost and lower energy costs.

http://www.eca.gov.uk/etl/about/Value+and+Benefit.htm

Finding Eligible Technologies: This link provides the information on the various technologies that are currently out there that an ECA can be claimed for. http://www.eca.gov.uk/etl/find/

There are a few technologies that are not listed as they are very variable or to numerous in type. More information on how to check if the technology is eligible for a company to claim an ECA is provided in the link below. In many cases if a contractor is hired to complete the work they should be able to provide information on whether or not an ECA can be claimed.

http://www.eca.gov.uk/etl/claim/non-listed.htm

Claiming an ECA: This link provides information on how and where to claim an ECA.

http://www.eca.gov.uk/etl/claim/

## Appendix B

Choosing the Right Electricity Company

This basic guide has been written in order to provide small businesses with a means to negotiate electricity prices. According to the figure below, gas prices and electricity prices are rising at an increasing rate. The figure shows average electricity prices from the company Powergen, one of the “Big 6” utility companies.

Figure 2: Quarterly comparison of electricity prices from Powergen

Though the plot only shows prices for Powergen, when one company increases their prices, the others usually follow in a relatively quick time period.

The worst way to pay for energy is through a non-contracted agreement, or tariff rate, with variable pricing for estimated units of electricity used. The best is a contract with a locked electricity rate for an agreed amount of time. Choosing the time period of the contract can be tricky because longer contracts will have a higher price per kilowatt hour (kWh) in an attempt to factor in the rising electricity prices. The most common advice is to go with the longer contract unless the price is significantly higher than the short-term rate.

There are two types of billing options for a contracted electricity agreement, these are debit and quarterly bills. Debit billing, usually the cheaper option, allows the energy user to pay a fixed rate every month, while quarterly billing allows the user to pay a variable bill, bill dependent upon energy use, every quarter.

These billing statements include two types of fees, the price per unit and the standing price. The standing price is a charge per day and can vary based on the electricity company, but should not be the major factor in making a decision. The price per unit is how much an electricity company charges for a kWh. Since not all energy companies raise their prices at the same time, shopping around is very important.

Energy companies will allow separate pricing for electricity used during the day and electricity used at night, known as Economy 7 or Economy 10, provided the correct meter is installed. Generally, Economy 7 is most beneficial when less than 75% of the total electricity used in total is during the day. The typical hour for which Economy 7 starts is 21.00-6.00, 0.00-7.00 or 1.00-8.00, this depends on service area and energy provider. Be sure to ask an energy consultant about both Economy 7 and Economy 10 and whether it is right for you or not. For more information on Economy 7 please visit: http://www.uswitch.com/gas-electricity/economy-7

The best type of meter for Economy 7 pricing is a smart meter, which takes half-hourly or hourly readings. By the year 2020, all businesses will be required to have smart meters. Usually these are not free but can be paid for incrementally through the standing charge portion of an electricity bill. Since smart meters cost around £100, an increased standing charge of 30p a day will accumulate to a smart meter after one year. This smart meter will be purchased by the building owner and will belong to them, not the electricity company. The purchased meter can be used if the electricity company is changed, provided the new electricity company supports smart metering.  Most of the “Big 6” electric companies support smart metering, be sure to ask upon calling. Most of these major electricity companies also do a combined gas and electricity bill. Be sure to ask if this lowers the price at which energy is purchased.

A list of the biggest electric companies has been provided below to facilitate comparative pricing.

| Company Name | Phone Number |
| --- | --- |
| E.ON | 0800 051 5517 |
| Powergen | 0800 051 0760 |
| British Gas | 0800 480 0202 |
| EDF | 0845 366 3664 |
| Scottish and Southern Energy | 0808 156 0056 |
| Npower | 0845 270 0926 |
| Scottish Power | 0800 980 2476 |

A list of impartial websites for advice on finding a cheaper company or energy saving advice is also provided.

| Website Name | Website Address |
| --- | --- |
| Consumer Focus: Energy Watch | http://www.energywatch.org.uk/ |
| Surrey Business Link | http://www.businesslink.gov.uk/south_east.html |
| uSwitch | http://www.uswitch.com/electricity-gas/ |

After research on comparative prices has been done, a free energy consultant can be contacted. They will find the cheapest electricity available and use the prices you found to possibly find better deals.

| Company Name | Phone Number |
| --- | --- |
| Business Advisory Service | 0845 180 0700 |
| Blue Mark Consultants | 0800 987 5505 |
| Quaestor Cost | 0800 970 0432 |
| uSwitch | 0800 051 5492 |

Figure from: http://uk.theoildrum.com/uploads/465/cv_powergen_gas_elec.png