A Study of the Effects of Surface Roughness on Amount of Heat Transferred

A Major Qualifying Project Report:

Submitted to the faculty of

Worcester Polytechnic Institute

In partial fulfillment of the requirements for the

Degree of Bachelor of Science

In Mechanical Engineering

Submitted by:

Michael R. Ayer

Approved by:

Professor Christopher A. Brown, Advisor

Surface Metrology Laboratory

Mechanical Engineering Department

Worcester Polytechnic Institute

Worcester, MA 01609

April 2010

Keywords: Surface Metrology, Heat Transfer

Table of Contents

Abstract	3

Introduction	4

Objective	4

Rationale	4

Solar hot water heaters	4

State-of-the-art	4

Approach	5

Methods	6

Test Box	6

Test Apparatus	7

Surface Texture Preparations	7

Surface Roughness Measurements and Area-scale Analysis	8

Testing Using the Apparatus	8

Measure Temperature Using LabView	8

Results	10

Surface Processing	10

Surface Measurements	10

Temperature Test Results	10

Discussion	12

Surface Processing	12

Surface Measurements	12

Temperature Tests	12

Conclusion	13

Acknowledgements	13

References	13

# Abstract

Heat transfer is composed of convective, conductive and radiant heat transfer.   There are many variables that can affect heat transfer: size, type of material, reflectivity, etc.  Aluminum is a readily available test material that has very well documented properties and an easily changed surface texture.  A silicon based sand paper was used to polish the test materials.  The surface textures of the aluminum samples were measured using an Olympus LEXT 3100 scanning laser microscope.  Area-scale analysis was also used to determine at which scale the most heat transfer was observed.

# Introduction

## Objective

In heat transfer, whether it be conductive, convective, or radiant, area affects the amount of heat transferred.  As the area of the material changes, the amount of heat transferred changes as well.  In the case of radiant heat transfer, the term emissivity is also affected by surface texture.  This research analyzed the hypothesis that modifying the surface texture would result in an increase in the amount of heat transferred.

By analyzing the final temperatures of the test materials with varied surface textures after being exposed to a work lamp for thirty minutes, we hope to be able to determine the scale at which the most heat was transferred.

## Rationale

## Solar hot water heaters

Aluminum is used in many applications that involve solar power.  One specific example is solar hot water heating. in some solar hot water heaters an aluminum plate is used to absorb heat from the sun and transfer it to water being stored in a reservoir.  If one could increase the amount of heat being transferred to the water in the reservoir, the system would become more efficient.  The more efficient the system , the more money the person or business using that system saves.

## State-of-the-art

Not much research has been done on the correlation between surface roughness and heat transferre.  In a literary search for previously conducted research involved  many different scholarly search engines, such as Lexis Nexis, Engineering Village, Science Direct, and Jstore.  The search terms included: surface texture, surface texture and heat transfer, surface texture and absorptivity, surface texture and radiant heat transfer, and texture gradient and heat transfer.

## Approach

Since there has not been much research in this area a specific test and procedures had to be devised.   This test procedure was based on similar test procedures that are documented in ASTM E-744.  Changes to E-744 were made due to the scale of the test being performed and the costs of acquiring the test lamp. ASTM standard E-744 requires the use of a Xenon Arc lamp, which is expensive and hard to obtain.  The use of a Halogen work lamp was suggested to me by George Kelly of BP solar and was used in this research.  The test material used is T6061 aluminum, and its surface texture was changed using silicon based sand paper.  Measurements of the surface were taken using an Olympus LEXT 3100 scanning laser confocal microscope.  The lamp was used to heat up the test material and LabView was used to record temperature in relation to time.  This study used area-scale analysis to compare relative area with respect to scale.

# Methods

## Test Box

The test box is an aluminum box that holds the test specimen.  The box is covered with a layer of Lexan to allow the box temperature to be mostly affected by the light.  The box is also painted black to help absorb as much heat from the light as possible.  Figure 1 below is an illustration of the box suggested by ASTM E-744. Some modifications to the box were made to fit my research.  Test specimens were elevated off the bottom of the test box using microscope slides.  This cut down on the amount of convective heat being transferred.  The test used  two thermocouples instead of one; one thermocouple was used to obtain the temperature of the test specimen while the second thermocouple was used to record the temperature of the inside of the box.  Again, this thermocouple was elevated off the bottom of the box to cut down on convective heat being transferred from the box.  A piece of aluminum foil was used to shield the thermocouple from the light.  This was to block the light from shining directly on the thermocouple, as this could have increased the recorded temperature due to heat being transferred from the light to the thermocouple through radiation.

Figure 1: Test Box ASTM E-744

## Test Apparatus

An adapted test method was designed based on  ASTM standard E-744. In standard E-744 a test box was prepared with a cover to house the test materials. Then one could choose to do an outdoor test or an indoor test.  The test apparatus was based off of the indoor test suggested by E-744.  A Xenon arc lamp was used in E-744 as the heating source.  After contacting ASTM and then being forwarded to George Kelly at BP Solar, I was informed that a cheaper and easier to obtain heat lamp would suffice for the test.  A Halogen Work lamp with a 300 watt bulb was used as a heat source in my experiments..  The light was hung 6 inches from the test box and was angled to face down directly on the test box.  To take temperature data a DAQ was used.  This DAQ converted the analog data being obtained by the thermocouples to digital data that could be read by my computer.  The DAQ was set up to measure the temperature of the test specimen while at the same time measuring the temperature of the box itself.  The test apparatus was then housed in a dark room where no outside light could affect the test specimen.

## Surface Texture Preparations

The test specimens were 2 inches long, 2 inches wide, and .5 inches thick.  The surfaces of the test specimens were prepared using silicon based sand paper.  The sand paper was used to polish the aluminum. The control specimen had no surface texture modifications made.  The grits of the sand paper used were: 120, 240, 400 and 600. Pictures of the test specimens are presented in the appendix.  Preparations to the surface were conducted by hand. This was to attempt to make the surfaces the same and help with repeatability.  First the use of used the roughest grit, 120, to polish the aluminum past the extrusion lines.  The extrusion lines for the control were left intact (see the appendix).  Once past the extrusion lines, the sand paper was changed to a finer grit periodically until reaching the final stage. Then used isopropyl alcohol was used to clean the surfaces of dirt and particles that could have been left behind by the sandpaper.  After cleaning the specimens their weights were measured and recorded

## Surface Roughness Measurements and Area-scale Analysis

All surface roughness measurements were conducted using an Olympus LEXT 4000 laser scanning confocal microscope. The fine acquisition setting was used to obtain all surface roughness data. All surface roughness measurements were obtained using 50X-magnification.  After obtaining topographical images of the surfaces Then files were converted to a format that could be analyzed by the software SFRX.  SFRX was used to complete the area-scale analysis. The area-scale analysis used, as suggested by my advisor, was bottom-left.

## Testing Using the Apparatus

Before testing, each test specimen and the apparatus cover plate was cleaned. The cover plate needed to be cleaned carefully to avaoid scratching it.  After connecting the thermocouples to the test specimen and the test box and connecting the DAQ to the computer recording the data;the room with the test apparatus was sealed offwith all lights except the light source off was the beginning of each test. .  All tests began at room temperature and be exposed each sampleto the heat source for only 30 minutes.  A thermocouple was attached to the bottom of each test specimen using a hot glue gun.  Then the test specimen was placed on the test stand.  The thermocouple that measures the box temperature was fixed to the bottom of the box but the junction was raised off the bottom and the thermocouple was shielded from direct exposure to the light source by aluminum foil.

## Measure Temperature Using LabView

To measure the temperature data of the test, a SCXI-1100 module from National Instruments was used.  This multi channel DAQ was perfect for taking two temperature measurements at the same time while also providing a time stamp for when the data was measured.  However, programming LabView to take multiple samples from multiple channels took longer than expected.  The figure below illustrates the block diagram of the VI used to take my temperature measurements.  then LabView wrote all data points to Excel where the analysis was conducted

Figure 2: LabView Block Diagram

# Results

## Surface Processing

The surfaces of the test specimens were prepared using different grit sand papers and manually polishing the specimen to the desired state.  Using the sand paper proved to be effective but time consuming.  The process was also not uniform; it was difficult to produce the same texture throughout the test material.  The inconsistent texture of the test materials had an effect on the results of the experiment.  Also while polishing a test sample using SP600, I noticed a discoloration in the material.  During polishing the test material became darker.  This also affected my results due to more energy being abosrobed by the darker material.  The test material SP600 was considered an outlier.  All test materials were weighed after polishing.

## Surface Measurements

Using the Olympus LEXT 4000 to measure the surface, data was recorded for all test specimens.  This was done using the 50X objective lens. The objective lens used was determined by measuring the roughness using 5X, 10X, 20X, and 50X.  After comparing the data from the measurements it was decided that the 50X provided the most accurate readings.  A graph of the combined area-scale analysis is provided in the Appendix.  I then measured all test specimens with the 50X objective lens and compared the measured roughness.  In the Figure below is the area-scale graph of all test specimens at 50X.

## Temperature Test Results

Prior to testing, all test specimens were cleaned and initial temperatures were recorded.  The Figure below displays temperature-time plots for all test specimens.

Figure 3: Temp-Time

Each test did not start at the same temperature. The Figure below displays the change in Temperature-time plots for all test specimens

Figure 4: dTemperature-Time

# Discussion

## Surface Processing

Since the test specimens were prepared by hand there were some variances in the surface texture, most notably the corners.  In future tests  chamfering the edges will help make an even surface for each test specimen.  This experiment did not consider test specimens that were prepared with CnC machines.  In future tests I would suggest machining the surface of each test specimen with different end mills, noting the feet rates and step-overs for repeatability.  It is important to have an even surface texture.

## Surface Measurements

After compiling surface measurements from SP120 using the 5X, 10X, 20X, and 50X objective lens, i decided that the 50X objective lens was best suited to take measurements.

## Temperature Tests

A difference in final temperature was observed between the test specimens.  Of the test specimens, SP600 had the largest difference in temperature. This is most likely due to darkening of the test specimen during surface polishing.  The specimen with the second highest temperature difference was test specimen Normal.  There is a noticeable difference in how fast the temperature rose and the final temperature of the specimen after 30 minutes.  In future studies, analysis could be performed to calculate the total amount of heat transferred.  This can then be used to obtain the amount of radiant heat transferred.  After obtaining the amount of radiant heat transferred, one could then study how surface texture affects emissivity. “Properties of emissivity strongly depend on surface conditions… This uncertainty is largely due to the difficulty in characterizing and describing the surface conditions precisely”. (P.889 TFS)  Also further analysis could be done pertaining surface texture and its effects on total amount of heat transferred.  In the Appendix are figures that display obtained temperature-time data and potential analysis of heat transferred.

# Conclusion

- Silicon based sand paper can be used to change the surface texture of aluminum to test.  Pre-surface-processing techniques, such as chamfering of edges, can be implemented to promote an even surface texture for more accurate results.

- The higher the magnification of the objective lens the more accurate the area-scale analysis.

- Changing the surface texture of aluminum causes a change in the amount of heat transferred.  The more rough the surface is the more heat that is transferred.

# Acknowledgements

The author would like to thank his advisor, Professor C. A. Brown, for his knowledge, support and guidance,  Olympus for the access to LEXT4000, and  Brendan Powers for help with the lab and SFRX.  Finally, I would like to thank Worcester Polytechnic Institute for providing me with a world class education.

# References